Procedure

Explosives Management

[Enter Project Name]

| Document Number | Revision | Date |
| --- | --- | --- |
| RTPR-HSE-PRO-XXXX | Rev 0 | 30/06/2026 |

| Status | Name | Position | Signature |
| --- | --- | --- | --- |
| Originator | Ashleigh Phillips | [External] HSES Industry Partners |  |
| Reviewed | Kirsten Snyman | Snr Manager, CSP RioProjects |  |
| Reviewed | Matt Moroz | Snr Manager, HSES RioProjects |  |
| Approved | Greg Doyle | Vice President HSESC, SD&T |  |

Contents

0	How to Use This Document	3

0.1	Purpose of This Template	3

0.2	How This Document is Structured	3

0.3	Step-by-Step Completion Instructions	3

0.4	Pre-Issue Checklist	4

0.5	Document Relationships	4

1	Heading	6

Heading

INSTRUCTION: Read, action and delete this box before this procedure is issued. Delete the entire Section 0 before this document is submitted for review or approval. It must not appear in any approved version of this procedure.

0  How to Use This Document

This section explains the purpose of the template, how it is structured, the sequence for completing it and the checks required before it is issued. It must be read in full before any section of this document is completed. Delete Section 0 in its entirety before issue.

0.1  Purpose of This Template

This template supports the preparation of a project-specific Explosives Management Procedure for projects delivered under an Owner-Led model. Under this model, Rio Tinto directly manages all project activities, and this procedure applies to all personnel on the project, including Rio Tinto staff and all contractor organisations.

This template is provided as a 90% draft. Fixed content reflects the minimum Rio Tinto requirement and must not be reduced. The remaining 10% covers role names, register locations, schedules and project-specific triggers, and must be completed by the project team before issue.

0.2  How This Document is Structured

This document contains four content types:

Fixed content is plain black text reflecting minimum Rio Tinto requirements. Fixed content must not be reduced, deleted or overridden.

[INSERT: placeholder text] appears in blue bold. Each placeholder must be replaced with accurate, project-specific information before issue.

Orange text marks specific who, what, when, or how detail included as a best practice suggestion. The project team may retain, modify, or replace orange detail.

0.3  Pre-Issue Checklist

Before submitting this document for review or approval, confirm all items below are complete, then delete this checklist together with the rest of Section 0.

| Check | Complete? |
| --- | --- |
| Cover page is complete including document number, revision, date and signatories | ☐ |
| All [INSERT:] placeholder text has been replaced with project-specific information | ☐ |
| All instruction boxes have been read, actioned and deleted | ☐ |
| All role titles match the actual project organisational structure | ☐ |
| All system and register locations are confirmed and inserted | ☐ |
| All workshop and meeting dates are confirmed against the project programme | ☐ |
| Related procedures in the References section are issued or being issued concurrently | ☐ |
| Section 17 (Source Requirements Traceability) will be deleted before final issue | ☐ |
| Section 16 (Project Localisation Edits) has been populated with all changes made during preparation | ☐ |
| Section 0 has been deleted in its entirety | ☐ |

1  Purpose and Scope

This section defines the purpose of this procedure, the project lifecycle phases within scope, and the obligations it establishes from project Execution through to Handover.

1.1  Purpose of This Document

This section states the purpose of this procedure and the requirements it establishes for the project.

This procedure defines requirements for the safe storage, transport, handling, use, and management of explosives and explosive-related materials on or for the owner-led project. All explosives-related work requires prior authorisation. The Contractor must comply with all local rules and regulations governing explosives, and all explosives activities must be conducted by competent and authorised personnel holding current statutory licences.

1.2  Scope of This Document

This section defines which activities and lifecycle phases are governed by this document.

This procedure applies to all Rio Tinto project personnel and all contractor organisations performing work under an Owner-Led project. It governs explosives management from project Execution through to Handover.

[INSERT: scope qualifications] — note any activities, phases, or locations excluded from this procedure's scope

2  Roles and Responsibilities

This section defines the responsibilities for all project roles relevant to this procedure, covering the Rio Tinto project team and all contractor organisations. Responsibilities are aligned to governance, oversight, assessment execution, reporting, and control verification.

| Responsibilities | Company | Contractor | Section |
| --- | --- | --- | --- |
| Project Management |  |  |  |
| Holds overall accountability for project compliance with this procedure and approves this procedure and any material amendment. | ✓ |  |  |
| Accepts residual risk at the highest risk class level and authorises continuation where escalation is required. | ✓ |  |  |
| [INSERT: add further responsibilities as required] |  |  |  |
| HSE/CSP Management |  |  |  |
| Maintains and implements this procedure and confirms all activities meet its requirements before work proceeds. | ✓ | ✓ |  |
| Reviews and approves all required assessments, plans, and registers produced under this procedure. | ✓ | ✓ |  |
| Reports performance against this procedure at the agreed project HSESC performance meeting. | ✓ | ✓ |  |
| Reviews performance data, identifies trends, and incorporates systemic findings into corrective action programmes. | ✓ | ✓ |  |
| [INSERT: add further responsibilities as required] |  |  |  |
| Frontline Supervision |  |  |  |
| Ensures work crew compliance with this procedure before and during all relevant task executions. | ✓ | ✓ |  |
| Stops work when the requirements of this procedure cannot be met and notifies the HSESC Manager. | ✓ | ✓ |  |
| Confirms all personnel under their supervision are trained and competent before commencing relevant activities. | ✓ | ✓ |  |
| [INSERT: add further responsibilities as required] |  |  |  |
| All Project Personnel / Workers |  |  |  |
| Complies with the requirements of this procedure in all relevant work activities. | ✓ | ✓ |  |
| Reports any non-compliance with this procedure immediately to their supervisor. | ✓ | ✓ |  |
| [INSERT: add further responsibilities as required] |  |  |  |

3  Regulatory Compliance and Authorisation

This section defines the regulatory authorisation requirements that must be satisfied before any explosives activity commences on the project, covering licences, approvals, and storage area assessments.

All explosives licences and regulatory approvals required by Applicable Laws must be obtained before any explosives are transported to, stored on, or used at the project site. Copies of all licences must be held on site and provided to the Company upon request. Compliance with all local rules and regulations governing explosives is required throughout the life of the project — refer RTPR-HSE-PRO-0001.

The table below records the licences and approvals required for this project. All entries must be completed and confirmed before any explosives are transported to site:

Table 1: Explosives Licences and Regulatory Approvals

| Licence / Approval Type | Issuing Authority | Licence Number | Issue Date | Expiry Date | Held On Site (Y/N) |
| --- | --- | --- | --- | --- | --- |
| Explosives storage licence | [INSERT: issuing authority] | [INSERT: licence number] | [INSERT: date] | [INSERT: date] | [INSERT: Y/N] |
| Explosives use / blasting licence | [INSERT: issuing authority] | [INSERT: licence number] | [INSERT: date] | [INSERT: date] | [INSERT: Y/N] |
| Dangerous goods transport approval | [INSERT: issuing authority] | [INSERT: licence number] | [INSERT: date] | [INSERT: date] | [INSERT: Y/N] |
| Ammonium nitrate storage approval | [INSERT: issuing authority] | [INSERT: licence number] | [INSERT: date] | [INSERT: date] | [INSERT: Y/N] |
| Detonator storage licence | [INSERT: issuing authority] | [INSERT: licence number] | [INSERT: date] | [INSERT: date] | [INSERT: Y/N] |
| [INSERT: additional licence type] | [INSERT: issuing authority] | [INSERT: licence number] | [INSERT: date] | [INSERT: date] | [INSERT: Y/N] |

PROJECT COMPLETION — ITEM 2: Identify and document all regulatory licences and approvals required for explosives activities in the project jurisdiction. Confirm that all licences are in place before any explosives are transported to site. Insert licence numbers and expiry dates into Table 1 above.

All dangerous goods storage areas including explosives, ammonium nitrate, and detonators must be assessed, approved, and appropriately licensed in accordance with Applicable Laws before use. No storage area may receive or hold explosive materials until the relevant approval is confirmed and recorded in Table 1.

4  Explosives Management Plan

This section defines the requirement for a site-specific Explosives Management Plan and prescribes the minimum content that plan must address for this project.

An Explosives Management Plan must be developed and maintained for this project, covering all phases of explosives activity from first receipt to final reconciliation and demobilisation. The plan must be submitted to the Company for review and approval before any explosives are brought to site.

PROJECT COMPLETION — ITEM 1: Confirm whether explosives are within scope for this project. If yes, identify all explosives activities planned, the types of explosives and detonators to be used, and the quantities. Insert this information into the Explosives Management Plan.

PROJECT COMPLETION — ITEM 3: Develop the site-specific Explosives Management Plan for this project covering all sections required by this procedure. Submit to the Company for review before any explosives are brought to site.

The Explosives Management Plan must address, as a minimum, the following elements:

Transport of explosives to site and on-site movement, including vehicle requirements, routes, and separation of explosives from detonators

Storage of explosives and detonators, including magazine construction, positioning, and access controls

Controlled access to explosive storage areas, including personnel authorisation and key management

On-site manufacture and mixing where applicable, including risk assessment and documented procedure requirements

Handling and preparation for use, including competency requirements for shot firers and explosives handlers — refer RTPR-HSE-PRO-0004

Blast design and initiation, including blast exclusion zone risk assessment — refer RTPR-HSE-PRO-0002

Management of misfires, including immediate response, waiting times, and reporting obligations

Reconciliation, transfer of ownership, and quality control, including daily stock records and signed transfer documentation

Record-keeping, including retention periods for all explosives records

Emergency response for explosive incident scenarios — refer RTPR-HSE-PRO-0041

[INSERT: additional_emp_elements]

Table 2: Explosives Activities and Materials Planned for This Project

| Explosives Activity | Explosive Type / Product | Detonator Type | Estimated Quantity | Activity Phase |
| --- | --- | --- | --- | --- |
| [INSERT: activity, e.g. bulk excavation blasting] | [INSERT: product name] | [INSERT: detonator type] | [INSERT: quantity and unit] | [INSERT: project phase] |
| [INSERT: activity] | [INSERT: product name] | [INSERT: detonator type] | [INSERT: quantity and unit] | [INSERT: project phase] |

5  Transport

This section defines the transport requirements for all movements of explosives to site and within the site, including vehicle, personnel, and documentation requirements.

All transport of explosives to site and all on-site movement of explosives must comply with applicable dangerous goods transport regulations. A documented transport risk assessment must be completed and approved before any explosives movement commences — refer RTPR-HSE-PRO-0002.

All vehicles transporting explosives must meet the following requirements:

The vehicle must be appropriate for the type and quantity of explosive load being carried

The vehicle must be fitted with all safety equipment required by applicable dangerous goods transport regulations

Required safety equipment includes, as a minimum: fire extinguisher, first aid kit, approved hazardous materials placards, and emergency response guide

The vehicle must be operated by personnel who hold the relevant dangerous goods transport licence or certification required in the project jurisdiction — refer RTPR-HSE-PRO-0004

Explosives and detonators must be transported separately in compliance with regulatory separation requirements and must never be carried in the same compartment or on the same vehicle unless permitted by Applicable Laws and approved in the transport risk assessment

On-site movement of explosives must follow approved routes documented in the Explosives Management Plan. The shot firer or authorised explosives handler supervises all on-site movements and confirms delivery quantities against the daily stock reconciliation record before explosives are placed in storage or taken to the blast area.

Personnel involved in the transport of explosives must raise a permit via the site PTW system before commencing work, refer RTPR-HSE-PRO-0021.

6  Storage

This section defines the construction, positioning, access control, and daily reconciliation requirements for all explosive magazines and storage areas on this project.

All explosive magazines and storage areas must be constructed, positioned, and operated in compliance with Applicable Laws and the conditions of the storage licence issued for this project. No magazine or storage area may be used until it has been inspected and approved in accordance with the relevant regulatory requirements.

The following access and storage controls apply to all explosive storage areas on this project:

Access is restricted to authorised personnel only, as listed in the current authorised access register held at the magazine

The authorised access register is reviewed and updated by the Shot Firer in Charge at the start of each project phase and whenever personnel authorisations change

Magazine keys are controlled by the Shot Firer in Charge and are not left unattended or transferred to unauthorised personnel

Explosives and detonators are stored in separate magazines in accordance with regulatory separation requirements

Barricading and exclusion zones around storage areas are established and maintained in accordance with RTPR-HSE-PRO-0028

A daily stock reconciliation must be conducted for all explosives and detonators held in storage. Any discrepancy identified during reconciliation must be reported immediately to the Company and to the relevant regulatory authority. The reconciliation process is defined in Section 8 of this procedure.

Table 3: Explosive Storage Areas for This Project

| Storage Area ID | Location on Site | Contents (Explosive Type) | Approved Capacity | Licence / Approval Reference | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| [INSERT: magazine ID] | [INSERT: grid reference or description] | [INSERT: explosive type] | [INSERT: max quantity and unit] | [INSERT: licence reference] | [INSERT: role title] |
| [INSERT: detonator store ID] | [INSERT: grid reference or description] | Detonators | [INSERT: max quantity and unit] | [INSERT: licence reference] | [INSERT: role title] |

7  On-Site Manufacture and Mixing

This section defines the controls that apply where explosives are manufactured or mixed on site, including risk assessment, procedural approval, and regulatory compliance requirements.

Where on-site manufacture or mixing of explosives is conducted, a specific risk assessment and a documented procedure approved by a competent person must be in place before any manufacture or mixing commences. On-site manufacture must comply with all applicable regulatory requirements and licencing conditions — refer RTPR-HSE-PRO-0002 for the risk assessment process.

The documented on-site manufacture procedure must address, as a minimum, the following:

The specific product or mixture to be manufactured and the quantities permitted per batch

Competency requirements for all personnel involved in manufacture or mixing — refer RTPR-HSE-PRO-0004

Equipment requirements and pre-use inspection requirements for all mixing equipment

On-site mixing units must be inspected by a competent person before each production run and the inspection recorded in the equipment log

Controls for managing incompatible materials during mixing, storage, and transport of mixed product

Quality control checks to be performed on the mixed product before use, with results recorded

Actions to be taken if an out-of-specification product is identified during or after mixing

Personnel involved in on-site manufacture or mixing must raise a permit via the site PTW system before commencing work, refer RTPR-HSE-PRO-0021.

PROJECT COMPLETION: Confirm whether on-site manufacture or mixing of explosives is within scope for this project. If yes, develop and attach the site-specific manufacture and mixing procedure and confirm that the required risk assessment has been completed and approved by a competent person before commencement.

8  Misfire Management

This section defines the documented process for responding to suspected and confirmed misfires, including re-entry controls, competency requirements, and mandatory reporting obligations.

A documented misfire management procedure must be in place before any blasting activity commences on this project. The procedure must be prepared by a competent person and approved before first blast. All personnel involved in blasting operations must be familiar with the misfire management procedure before commencing work.

The misfire management procedure must define, as a minimum, the following:

The immediate response required upon identification of a suspected or confirmed misfire, including blast area re-entry prevention and the blast exclusion zone requirements — refer RTPR-HSE-PRO-0028

Minimum waiting times before re-entry following a suspected misfire: 30 minutes for shock tube or electronic detonators and 60 minutes for electric detonators, or as required by Applicable Laws if more stringent

The step-by-step process for dealing with misfired holes, including identification, marking, isolation, and treatment or destruction of the misfired charge

Competency requirements for the personnel authorised to manage misfires in the specific regulatory jurisdiction — refer RTPR-HSE-PRO-0004

Mandatory reporting of all misfires to the Company and to the relevant regulatory authority within the timeframes required by Applicable Laws — refer RTPR-HSE-PRO-0009

PROJECT COMPLETION — ITEM 4: Confirm the misfire management procedure for this project, including the competency requirements for personnel authorised to manage misfires in the specific regulatory jurisdiction. Attach or reference the approved procedure in this section.

Table 4: Misfire Management — Key Project Parameters

| Parameter | Project-Specific Requirement |
| --- | --- |
| Minimum re-entry wait time (regulatory requirement) | [INSERT: time and regulatory reference] |
| Personnel authorised to manage misfires | [INSERT: role title(s) and licence/competency required] |
| Regulatory authority for misfire reporting | [INSERT: authority name and contact] |
| Reporting timeframe required by Applicable Laws | [INSERT: timeframe] |
| Misfire record retention period | [INSERT: period or refer to Section 8] |

All misfires must be reported and investigated in line with Company requirements — refer RTPR-HSE-PRO-0009.

9  Reconciliation and Record-Keeping

This section defines the daily reconciliation process, transfer of ownership documentation, quality control records, and minimum retention periods for all explosives records on this project.

Daily reconciliation of all explosives and detonators is required. Reconciliation records must account for the quantity received, the quantity used, and the quantity remaining in storage at the end of each day. Any discrepancy must be reported immediately to the Company and to the relevant regulatory authority.

The following documentation requirements apply to reconciliation, transfer, and quality control:

Daily reconciliation records are completed and signed by the Shot Firer in Charge at the end of each working day

The daily reconciliation record is completed using the project Explosives Reconciliation Form and stored in the site explosives register at the end of each shift

Transfer of ownership of explosives or detonators between parties must be documented with signed records from both the transferring and receiving parties before physical transfer occurs

Quality control records, including batch numbers, product specifications, and condition assessments, must be maintained for all explosive products received and used

All explosives records must be retained for a minimum of five years, or for the period required by Applicable Laws if longer, whichever is greater

Table 5: Explosives Record Types and Retention Requirements

| Record Type | Completed By | Frequency | Storage Location | Minimum Retention Period |
| --- | --- | --- | --- | --- |
| Daily stock reconciliation | [INSERT: role title] | Daily | [INSERT: location] | 5 years or as required by Applicable Laws |
| Transfer of ownership record | [INSERT: role title] | Each transfer | [INSERT: location] | 5 years or as required by Applicable Laws |
| Quality control record | [INSERT: role title] | Each delivery / batch | [INSERT: location] | 5 years or as required by Applicable Laws |
| Misfire record | [INSERT: role title] | Each misfire event | [INSERT: location] | 5 years or as required by Applicable Laws |
| Transport risk assessment | [INSERT: role title] | Per route or change | [INSERT: location] | 5 years or as required by Applicable Laws |
| [INSERT: additional record type] | [INSERT: role title] | [INSERT: frequency] | [INSERT: location] | [INSERT: retention period] |

10  Incident Reporting

This section defines the incident reporting obligations for all explosives-related incidents and deviations from the approved explosives process, including regulatory notification requirements.

All incidents or deviations from the approved explosives process must be reported and investigated in line with Company requirements — refer RTPR-HSE-PRO-0009. This includes misfire events, unplanned detonations, explosives losses or discrepancies, and any condition that is not consistent with the approved Explosives Management Plan.

Explosives-related incidents must also be reported to the relevant regulatory authority within the timeframes required by Applicable Laws. The Project Manager or delegated representative is responsible for ensuring regulatory notifications are made and records of those notifications are retained.

Table 6: Explosives Incident Notification Requirements

| Incident Type | Internal Reporting Pathway | Regulatory Authority | Regulatory Notification Timeframe |
| --- | --- | --- | --- |
| Unplanned detonation | Refer RTPR-HSE-PRO-0009 | [INSERT: authority name] | [INSERT: timeframe required by Applicable Laws] |
| Confirmed misfire | Refer RTPR-HSE-PRO-0009 | [INSERT: authority name] | [INSERT: timeframe required by Applicable Laws] |
| Explosives theft or loss | Refer RTPR-HSE-PRO-0009 | [INSERT: authority name and police] | [INSERT: timeframe required by Applicable Laws] |
| Explosives transport incident | Refer RTPR-HSE-PRO-0009 | [INSERT: authority name] | [INSERT: timeframe required by Applicable Laws] |
| Fire in or near explosive storage | Refer RTPR-HSE-PRO-0009 | [INSERT: authority name] | [INSERT: timeframe required by Applicable Laws] |
| [INSERT: additional incident type] | Refer RTPR-HSE-PRO-0009 | [INSERT: authority name] | [INSERT: timeframe] |

11  Emergency Response

This section defines the emergency response planning requirements for explosive incident scenarios, including mandatory scenarios, testing obligations, and integration with the project emergency response plan.

A specific emergency response plan addressing potential explosive incident scenarios must be developed before any explosives are brought to site. The emergency response plan must be integrated into the overall project Emergency Response Plan — refer RTPR-HSE-PRO-0041.

The emergency response plan for explosive incidents must address, as a minimum, the following scenarios:

Unplanned detonation, including immediate response, casualty management, and site security

Misfire, including blast area isolation, re-entry controls, and treatment or destruction of the misfired charge

Explosives theft or loss, including notification to the Company, regulatory authority, and police

Explosives vehicle accident, including transport emergency procedures, spill management, and dangerous goods incident response

Fire in or near explosive storage, including evacuation, exclusion zones, and notification to emergency services and the regulatory authority

[INSERT: additional_emergency_scenarios]

The emergency response plan for explosive incidents must be tested before explosives are brought to site. Testing must include a desktop review and, where practicable, a field exercise involving all key response roles. The test must be documented and outcomes recorded, with any gaps addressed before explosives arrive.

PROJECT COMPLETION — ITEM 5: Confirm the explosives emergency response scenarios for this project and integrate them into the project ERP — refer RTPR-HSE-PRO-0041. Test the explosives emergency response before explosives arrive on site and record the test outcomes below.

Table 7: Explosives Emergency Response Plan — Testing Record

| Test Type | Scenarios Tested | Date Conducted | Conducted By | Gaps Identified | Gaps Closed (Y/N / Date) |
| --- | --- | --- | --- | --- | --- |
| Desktop review | [INSERT: scenarios] | [INSERT: date] | [INSERT: role title] | [INSERT: description or nil] | [INSERT: Y/N and date] |
| Field exercise | [INSERT: scenarios] | [INSERT: date] | [INSERT: role title] | [INSERT: description or nil] | [INSERT: Y/N and date] |

All explosive emergency response contacts, including the relevant regulatory authority, emergency services, Company crisis contacts, and the explosives supplier emergency line, must be documented in the project Emergency Response Plan — refer RTPR-HSE-PRO-0041.

12  Definitions and Acronyms

The following terms are used throughout this procedure. Where a term is also used in other project HSESC procedures, the definition below applies consistently across the system.

| Term | Definition |
| --- | --- |
| ANFO | Ammonium Nitrate Fuel Oil — a bulk explosive mixture commonly used in blasting operations, consisting of porous ammonium nitrate prills and fuel oil. |
| AN | Ammonium Nitrate — an oxidising agent and primary component of many commercial explosives and blasting agents used in mining and construction. |
| EMP | Explosives Management Plan — a site-specific documented plan that governs the procurement, transport, storage, handling, use, reconciliation, and disposal of explosives on a project site. |
| PTW | Permit to Work — a formal written authority system that controls high-risk work activities, including blasting operations, to ensure appropriate hazard identification, authorisation, and controls are in place before work commences. |
| HSESC | Health, Safety, Environment, Security and Community — the integrated management framework applied across Rio Tinto projects to manage risks and obligations in all these domains. |
| MSDS | Material Safety Data Sheet — a document providing detailed information on the properties, hazards, safe handling, storage, and emergency response measures for a hazardous substance, including explosives and explosive precursors. |
| SDS | Safety Data Sheet — the current internationally harmonised terminology for MSDS under the Globally Harmonised System of Classification and Labelling of Chemicals (GHS), providing chemical hazard and safe handling information. |
| GHS | Globally Harmonised System of Classification and Labelling of Chemicals — an internationally agreed system for communicating chemical hazard information through standardised labels and Safety Data Sheets. |
| BEZ | Blast Exclusion Zone — a defined area surrounding a blast site from which all personnel, vehicles, and equipment must be excluded during charging, firing, and post-blast inspection activities. |
| EM | Explosives Magazine — a purpose-built, approved storage facility designed and constructed to safely store explosives and detonators in accordance with regulatory requirements and separation distances. |
| MMU | Mobile Manufacturing Unit — a licensed vehicle-mounted system used for the on-site manufacture and delivery of bulk explosive products such as ANFO or emulsion directly into blast holes. |
| PPE | Personal Protective Equipment — specialised clothing and equipment worn by personnel to minimise exposure to hazards, including those associated with handling, transporting, and using explosives. |
| RT | Rio Tinto — the global mining and resources group under whose standards, procedures, and risk management framework this procedure is developed and applied. |
| RTPR | Rio Tinto Project Requirement — the document identifier prefix used for project-level HSESC management system procedures applicable to Rio Tinto owner-led projects. |
| OEM | Original Equipment Manufacturer — the manufacturer of equipment or machinery whose specifications, instructions, and safety requirements must be followed during operation and maintenance, including explosive delivery and mixing equipment. |
| DG | Dangerous Goods — substances or articles that pose a risk to health, safety, property, or the environment during transport, including explosives classified under applicable dangerous goods transport regulations. |
| ADG | Australian Dangerous Goods Code — the national code governing the transport of dangerous goods by road and rail in Australia, including classification, packaging, labelling, and transport requirements for explosives. |
| IMDG | International Maritime Dangerous Goods Code — an international regulatory framework governing the safe transport of dangerous goods, including explosives, by sea. |
| IATA | International Air Transport Association — the organisation that publishes Dangerous Goods Regulations (DGR) governing the safe transport of hazardous materials, including explosives, by air. |
| DGR | Dangerous Goods Regulations — the IATA regulatory framework specifying requirements for the classification, packaging, labelling, documentation, and transport of dangerous goods by air. |
| ERG | Emergency Response Guidebook — a guide used by first responders and emergency personnel for initial response to incidents involving dangerous goods, including explosives, during transport. |
| JHA | Job Hazard Analysis — a systematic process used to identify, assess, and control hazards associated with specific work tasks, including blasting and explosives handling activities. |
| SWMS | Safe Work Method Statement — a document that identifies high-risk construction and blasting work activities, the hazards associated with them, and the control measures to be implemented to manage those hazards. |
| KPI | Key Performance Indicator — a measurable value used to evaluate the effectiveness of explosives management activities including compliance, reconciliation accuracy, and incident rates. |
| NCR | Non-Conformance Report — a formal document used to record any deviation from specified explosives management requirements, regulatory obligations, or approved procedures. |
| ICP | Incident Command Post — a designated location at or near an explosives incident site from which emergency response activities are coordinated by the Incident Commander. |
| ERP | Emergency Response Plan — a documented plan that defines the roles, responsibilities, resources, and actions required to manage emergencies arising from explosives incidents on a project site. |
| CCTV | Closed Circuit Television — a video surveillance system used to monitor and secure explosives storage areas and magazines against unauthorised access and theft. |
| UN | United Nations — the international body whose numbering system and model regulations form the basis for the classification and transport requirements of dangerous goods, including explosives, across global jurisdictions. |
| [INSERT: add project-specific terms] | [INSERT: definition] |

13  References and Source Information

This section lists the internal and external documents this procedure relies on. This section remains in the procedure once published. Where a referenced document is updated, this procedure is reviewed to confirm the reference remains current.

| Document | Reference | Type |
| --- | --- | --- |
| Legal Requirements Procedure | RTPR-HSE-PRO-0001 | Internal procedure |
| Risk Management Procedure | RTPR-HSE-PRO-0002 | Internal procedure |
| Training Procedure | RTPR-HSE-PRO-0004 | Internal procedure |
| Incident Management Procedure | RTPR-HSE-PRO-0009 | Internal procedure |
| Permit to Work Procedure | RTPR-HSE-PRO-0021 | Internal procedure |
| Barricading Procedure | RTPR-HSE-PRO-0028 | Internal procedure |
| Business Resilience Procedure | RTPR-HSE-PRO-0041 | Internal procedure |
| Rio Tinto Risk Management Standard | RT Risk Management Standard | External RT |
| Rio Tinto Risk Management Standard — Appendix C, Section C-1: Consequence and Likelihood Criteria | RT Risk Management Standard Appendix C C-1 | External RT |
| Rio Tinto Health, Safety and Environment Standard | RT HSE Standard | External RT |
| Rio Tinto Explosives Management Standard | RT Explosives Management Standard | External RT |
| Rio Tinto Fatal Risk Standard — Explosives | RT Fatal Risk Standard — Explosives | External RT |
| Australian Dangerous Goods Code for the Transport of Dangerous Goods by Road and Rail | ADG Code (current edition) | External framework |
| Globally Harmonised System of Classification and Labelling of Chemicals | GHS (current revision) | External framework |
| IATA Dangerous Goods Regulations | IATA DGR (current edition) | External framework |
| International Maritime Dangerous Goods Code | IMDG Code (current amendment) | External framework |
| United Nations Recommendations on the Transport of Dangerous Goods — Model Regulations | UN Model Regulations (current revision) | External framework |
| AS 2187.1 — Explosives — Storage, Transport and Use: Storage | AS 2187.1 | External framework |
| AS 2187.2 — Explosives — Storage, Transport and Use: Use of Explosives | AS 2187.2 | External framework |
| Emergency Response Guidebook | ERG (current edition) | External framework |
| ISO 31000 — Risk Management: Guidelines | ISO 31000 | External framework |
| [INSERT: further referenced document, where applicable] | [INSERT: reference] | [INSERT: type] |

14  Project Localisation Edits

This section captures the project-specific edits made to this document during preparation. This section stays in the document once published and is updated at each document revision. Record all additions, deletions, and wording changes from the original template.

Records of wording changes

| Description of the Change | Section |
| --- | --- |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |

Records of new content additions

| Description of the Change | Section |
| --- | --- |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |

Records of content removal

| Description of the Change | Section |
| --- | --- |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |

REMOVABLE SECTION — Source Requirements Traceability. Delete this entire section before the document is issued for approval. It must not appear in any approved version of this document. Table A records requirements addressed in this procedure. Table B records requirements not fully addressed, with written justification for each.

15  Source Requirements Traceability (DELETE BEFORE ISSUE)

15.1  Requirements Addressed in This Procedure

| Section | Source | Requirement | Status |
| --- | --- | --- | --- |
| [INSERT: section] | Appendix C C-2 §59.1.1 | Requires all explosives licences and regulatory approvals to be obtained before any explosives are transported to, stored on, or used at the project site, and requires compliance with all local rules and regulations governing explosives throughout the life of the project. | Full |
| [INSERT: section] | Appendix C C-4 §92.5.3.1 | Requires that all dangerous goods storage areas including explosives, ammonium nitrate, and detonators are assessed, approved, and appropriately licensed in accordance with Applicable Laws before use. | Full |
| [INSERT: section] | Appendix C C-2 §59.1 | Requires development and maintenance of an Explosives Management Plan covering all phases of explosives activity on the project, including transport, storage, manufacture, blast design, misfire management, reconciliation, record-keeping, and emergency response. | Full |
| [INSERT: section] | Appendix C C-2 §59.1.4 | Requires a specific risk assessment and documented procedure approved by a competent person before any on-site manufacture or mixing of explosives commences, with compliance with all applicable regulatory requirements and licencing conditions. | Full |
| [INSERT: section] | Appendix C C-2 §59.1.6 | Requires a documented misfire management procedure covering immediate response, minimum waiting times, process for dealing with misfired holes, competency requirements, and mandatory reporting to the Company and relevant regulatory authority. | Full |
| [INSERT: section] | Appendix C C-2 §59.1.9 | Requires daily reconciliation of all explosives and detonators with signed transfer records, quality control records for all products, and retention of all explosives records for a minimum of five years or as required by Applicable Laws. | Full |
| [INSERT: section] | Appendix C C-2 §59.2.1 | Requires that all incidents or deviations from the approved explosives process are reported and investigated in line with Company requirements, and that explosives-related incidents are also reported to the relevant regulatory authority within the timeframes required by Applicable Laws. | Full |
| [INSERT: section] | Appendix C C-2 §59.3.1 | Requires a specific emergency response plan for explosive incident scenarios covering unplanned detonation, misfire, theft or loss, vehicle accident, and fire in or near explosive storage, tested before explosives are brought to site. | Full |

15.2  Requirements Not Fully Addressed in This Procedure

List any requirements from the external sources above that are not fully addressed in this document, with a specific written justification for each. Acceptable justifications: (a) the requirement is owned by a named standalone procedure cross-referenced here; (b) excluded by a specific business decision endorsed by project leadership, named here; or (c) addressed through an operational control outside this document, identified here.

| Requirement Not Covered | Source | Justification |
| --- | --- | --- |
| [INSERT: description of requirement not covered] | [INSERT: source and clause] | [INSERT: specific written justification] |