Procedure

Tools and Equipment

[Enter Project Name]

| Document Number | Revision | Date |
| --- | --- | --- |
| RTPR-HSE-PRO-XXXX | Rev 0 | 30/06/2026 |

| Status | Name | Position | Signature |
| --- | --- | --- | --- |
| Originator | Ashleigh Phillips | [External] HSES Industry Partners |  |
| Reviewed | Kirsten Snyman | Snr Manager, CSP RioProjects |  |
| Reviewed | Matt Moroz | Snr Manager, HSES RioProjects |  |
| Approved | Greg Doyle | Vice President HSESC, SD&T |  |

Contents

0	How to Use This Document	3

0.1	Purpose of This Template	3

0.2	How This Document is Structured	3

0.3	Step-by-Step Completion Instructions	3

0.4	Pre-Issue Checklist	4

0.5	Document Relationships	4

1	Heading	6

Heading

INSTRUCTION: Read, action and delete this box before this procedure is issued. Delete the entire Section 0 before this document is submitted for review or approval. It must not appear in any approved version of this procedure.

0  How to Use This Document

This section explains the purpose of the template, how it is structured, the sequence for completing it and the checks required before it is issued. It must be read in full before any section of this document is completed. Delete Section 0 in its entirety before issue.

0.1  Purpose of This Template

This template supports the preparation of a project-specific Tools and Equipment Procedure for projects delivered under an Owner-Led model. Under this model, Rio Tinto directly manages all project activities, and this procedure applies to all personnel on the project, including Rio Tinto staff and all contractor organisations.

This template is provided as a 90% draft. Fixed content reflects the minimum Rio Tinto requirement and must not be reduced. The remaining 10% covers role names, register locations, schedules and project-specific triggers, and must be completed by the project team before issue.

0.2  How This Document is Structured

This document contains four content types:

Fixed content is plain black text reflecting minimum Rio Tinto requirements. Fixed content must not be reduced, deleted or overridden.

[INSERT: placeholder text] appears in blue bold. Each placeholder must be replaced with accurate, project-specific information before issue.

Orange text marks specific who, what, when, or how detail included as a best practice suggestion. The project team may retain, modify, or replace orange detail.

0.3  Pre-Issue Checklist

Before submitting this document for review or approval, confirm all items below are complete, then delete this checklist together with the rest of Section 0.

| Check | Complete? |
| --- | --- |
| Cover page is complete including document number, revision, date and signatories | ☐ |
| All [INSERT:] placeholder text has been replaced with project-specific information | ☐ |
| All instruction boxes have been read, actioned and deleted | ☐ |
| All role titles match the actual project organisational structure | ☐ |
| All system and register locations are confirmed and inserted | ☐ |
| All workshop and meeting dates are confirmed against the project programme | ☐ |
| Related procedures in the References section are issued or being issued concurrently | ☐ |
| Section 17 (Source Requirements Traceability) will be deleted before final issue | ☐ |
| Section 16 (Project Localisation Edits) has been populated with all changes made during preparation | ☐ |
| Section 0 has been deleted in its entirety | ☐ |

1  Purpose and Scope

This section defines the purpose of this procedure, the project lifecycle phases within scope, and the obligations it establishes from project Execution through to Handover.

1.1  Purpose of This Document

This section states the purpose of this procedure and the requirements it establishes for the project.

This procedure defines requirements for ensuring all tools and equipment used on the owner-led project are fit for purpose, properly maintained, inspected, and compliant with applicable safety standards — including a clear statement of what is banned and what requires approval before use.

1.2  Scope of This Document

This section defines which activities and lifecycle phases are governed by this document.

This procedure applies to all Rio Tinto project personnel and all contractor organisations performing work under an Owner-Led project. It governs tools and equipment from project Execution through to Handover.

[INSERT: scope qualifications] — note any activities, phases, or locations excluded from this procedure's scope

2  Roles and Responsibilities

This section defines the responsibilities for all project roles relevant to this procedure, covering the Rio Tinto project team and all contractor organisations. Responsibilities are aligned to governance, oversight, assessment execution, reporting, and control verification.

| Responsibilities | Company | Contractor | Section |
| --- | --- | --- | --- |
| Project Management |  |  |  |
| Holds overall accountability for project compliance with this procedure and approves this procedure and any material amendment. | ✓ |  |  |
| Accepts residual risk at the highest risk class level and authorises continuation where escalation is required. | ✓ |  |  |
| [INSERT: add further responsibilities as required] |  |  |  |
| HSE/CSP Management |  |  |  |
| Maintains and implements this procedure and confirms all activities meet its requirements before work proceeds. | ✓ | ✓ |  |
| Reviews and approves all required assessments, plans, and registers produced under this procedure. | ✓ | ✓ |  |
| Reports performance against this procedure at the agreed project HSESC performance meeting. | ✓ | ✓ |  |
| Reviews performance data, identifies trends, and incorporates systemic findings into corrective action programmes. | ✓ | ✓ |  |
| [INSERT: add further responsibilities as required] |  |  |  |
| Frontline Supervision |  |  |  |
| Ensures work crew compliance with this procedure before and during all relevant task executions. | ✓ | ✓ |  |
| Stops work when the requirements of this procedure cannot be met and notifies the HSESC Manager. | ✓ | ✓ |  |
| Confirms all personnel under their supervision are trained and competent before commencing relevant activities. | ✓ | ✓ |  |
| [INSERT: add further responsibilities as required] |  |  |  |
| All Project Personnel / Workers |  |  |  |
| Complies with the requirements of this procedure in all relevant work activities. | ✓ | ✓ |  |
| Reports any non-compliance with this procedure immediately to their supervisor. | ✓ | ✓ |  |
| [INSERT: add further responsibilities as required] |  |  |  |

3  Purpose and Scope

This section states the purpose of this procedure, the equipment categories it governs, and the project lifecycle phases to which it applies from Execution through to Handover.

3.1  Purpose of This Document

This section defines what this procedure establishes, the standards it operationalises, and the obligations it places on all project personnel.

This procedure meets the following purpose requirements:

This procedure governs the selection, inspection, certification, use, and defect management of all tools and equipment on the project, and applies to:

All Rio Tinto owner team personnel.

All contractor and subcontractor organisations working on the project.

All tools and equipment brought onto or used on the project site.

This procedure operationalises the requirements of the Rio Tinto HSESC Management System and applicable regulatory obligations into a documented, project-specific process for controlling hazards associated with tools and equipment.

This procedure defines absolute prohibitions, restricted item approval requirements, inspection and certification standards, and defect management obligations. Competency requirements for equipment inspection are governed by the Training Procedure, RTPR-HSE-PRO-0004.

3.2  Scope of This Document

This section defines which tool and equipment categories, project phases, and site locations are governed by this procedure.

This procedure applies the following scope requirements:

This procedure governs all portable hand tools, power tools, lifting equipment, electrical equipment, and associated accessories used during the following project phases:

Construction and Execution.

Commissioning.

Handover and Close-out.

This procedure does not govern the design or procurement specifications of fixed plant and permanent electrical installations. Those requirements are addressed in the relevant engineering and commissioning procedures.

4  Banned and Restricted Items

This section establishes absolute prohibitions on specified tools and equipment and defines the approval process for restricted items that require documented authorisation before use.

4.1  Banned Items

This section lists the items that are prohibited on the project site under all circumstances and without exception.

The following items are banned on the project site and must not be used under any circumstances. No approval process exists for these items. Any banned item identified on site must be removed from the work area immediately and reported to the HSESC team.

Banned items on this project are:

Man-made or improvised tools of any kind.

Over-centre type binders.

9-inch (230 mm) angle grinders.

Sit harnesses (waist-only harnesses).

Saferight auto-stop rope descender/brake devices.

Manual torque multipliers.

[INSERT: additional_banned_items]

Confirm whether the project scope, location, or specific activities require any additional items to be added to the banned list above. Insert project-specific additions in the placeholder provided. Some project locations or regulatory jurisdictions impose prohibitions beyond the standard Rio Tinto list.

4.2  Restricted Items

This section lists restricted items that require documented project owner approval before use and defines the approval process that applies.

The following items are restricted and may only be used on the project site after documented approval has been obtained from the project owner. Restricted items must not be brought onto site or used until the approval record is in place and has been communicated to the relevant work group.

Restricted items on this project are:

Open-bladed knives, including Stanley knives.

Skiving knives.

Unsecured scaffold hammer frogs.

Powered nail guns, whether explosive or pneumatic.

Hydro blasting equipment.

Personal music devices with headphones.

Handheld demolition or quick-cut saws.

Double-pointed podgy bars.

Hardened steel sledgehammers.

[INSERT: additional_restricted_items]

The approval process for restricted items applies the following requirements:

The restricted item approval process requires:

[INSERT: restricted_item_approval_authority]

A written approval request submitted by the contractor superintendent to the Construction Manager, identifying the item, the work task requiring its use, the duration of use, and the controls to be applied.

Approval documented on the Restricted Equipment Approval Form and filed in the project document management system before the item is mobilised to site.

Communication of the approval and associated controls to the work group at the pre-start briefing before the restricted item is used.

[INSERT: restricted_item_approval_record_location]

Define the project approval process for restricted items: confirm the role with approval authority; specify the documentation required; and confirm how approval is recorded and communicated to the workforce. Complete all placeholders in this section before issue.

5  Inspection and Certification

This section defines the inspection, certification, tagging, and pre-use check requirements for all safety-critical tools and equipment used on the project.

5.1  Lifting Equipment Certification

This section establishes the certificate requirements for all lifting equipment used on the project.

All lifting equipment used on the project, including cranes, hooks, and slings, must hold a valid and recognised test certificate confirming it is safe for operation. The certificate must be current at the time of use and must not have lapsed.

The responsible contractor superintendent makes all lifting equipment test certificates available to the project owner on request. Lifting equipment without a valid certificate must not be used and must be removed from service immediately.

Lifting equipment brought onto the project site is subject to verification by the HSESC team before first use. The verification check confirms the certificate is valid, the equipment matches the certificate description, and no visible defects are present.

5.2  Portable Electrical Equipment Inspection

This section defines the quarterly inspection, tagging, and registration requirements for all portable electrical equipment on the project.

All portable electrical equipment on the project site is inspected and tagged by a certified electrician on a quarterly basis. Equipment that does not carry a current inspection tag must not be used and must be removed from service pending inspection.

Inspection records are maintained in the Electrical Equipment Register, RTPR-HSE-REG-0010. Each entry in the register records the following:

The Electrical Equipment Register records for each item:

Equipment identification number.

Inspection date.

Certifier's name.

Next inspection due date.

Equipment is colour-coded following each inspection to indicate its status. Equipment carrying the current period colour code is acceptable for use. Equipment not carrying the current period colour code must be removed from service until inspected and re-tagged.

Table 1: Quarterly Electrical Equipment Inspection Colour Code Schedule.

| Quarter | Period | Colour Code | Approved Inspector |
| --- | --- | --- | --- |
| Q1 | January – March | {'insert': True, 'field': 'q1_colour', 'hint': 'Insert colour code for Q1'} | {'insert': True, 'field': 'q1_inspector', 'hint': 'Insert name and licence number of approved certified electrician'} |
| Q2 | April – June | {'insert': True, 'field': 'q2_colour', 'hint': 'Insert colour code for Q2'} | {'insert': True, 'field': 'q2_inspector', 'hint': 'Insert name and licence number of approved certified electrician'} |
| Q3 | July – September | {'insert': True, 'field': 'q3_colour', 'hint': 'Insert colour code for Q3'} | {'insert': True, 'field': 'q3_inspector', 'hint': 'Insert name and licence number of approved certified electrician'} |
| Q4 | October – December | {'insert': True, 'field': 'q4_colour', 'hint': 'Insert colour code for Q4'} | {'insert': True, 'field': 'q4_inspector', 'hint': 'Insert name and licence number of approved certified electrician'} |

Confirm the quarterly inspection schedule for this project and the arrangement for the approved certified electrician. Insert the colour codes and inspector details in the table above. Where a single project electrician holds this responsibility, confirm their name and licence number before issue.

5.3  Pre-Use Inspection of Safety-Critical Tools and Equipment

This section establishes the systematic pre-use inspection process and the defect management pathway for all safety-critical tools and equipment.

A pre-use inspection is completed for all safety-critical tools and equipment before each period of use. The inspection is conducted by the operator or the work group supervisor and confirms the item is free from visible defects, correctly configured, and within its service and certification period.

Any item identified as unsafe during a pre-use inspection is removed from service immediately. The operator or supervisor applies a defect tag to the item and places it in the designated quarantine area before work commences. Defective equipment incidents are reported per project requirements, refer RTPR-HSE-PRO-0009.

Competency requirements for equipment inspection are defined in the Training Procedure, RTPR-HSE-PRO-0004. Personnel who have not completed the required inspection competency must not conduct formal equipment inspections.

6  Defect Management and Tool Securing

This section defines the defect reporting and management process and the requirements for securing tools when working at height to prevent dropped objects.

6.1  Equipment Defect Reporting and Management

This section establishes the defect tag system, quarantine process, repair and replace authority, and re-inspection requirement before return to service.

The project applies an isolation tag system to control defective tools and equipment from the point of identification through to repair or replacement and return to service. No defective item is returned to service without completing the steps defined in this section.

The defect management process applies the following steps:

When a defect is identified, the following steps apply in sequence:

The operator or supervisor removes the item from service immediately and applies an isolation tag.

[INSERT: defect_tag_design]

The tagged item is placed in the designated defective equipment quarantine area located at the main site laydown area. The item must not be used or moved from the quarantine area without authorisation.

[INSERT: quarantine_area_location]

[INSERT: repair_replace_authority]

A re-inspection by a competent person is completed before the item is returned to service. The re-inspection confirms the defect has been rectified and the item meets the applicable standard. The item must carry a new inspection tag before being returned to service.

All defective equipment reports are recorded and, where the defect caused or could have caused an incident, reported per project requirements, refer RTPR-HSE-PRO-0009.

Define the project equipment defect tag system before issue: confirm the approved tag design and material; confirm the quarantine area location; and confirm the role with authority to approve return to service. Complete all placeholders in this section.

6.2  Tool Securing at Height

This section establishes the requirement to secure all tools when working at height and references the DROPS programme requirements.

All tools used at height must be secured using appropriate lanyards, tethering systems, or tool belts to prevent dropped objects. Tool securing is a mandatory control and applies to all personnel working at height on the project site, regardless of the duration of the task.

The requirements for tool securing, dropped object prevention systems, and DROPS programme compliance are defined in the Working at Height Procedure, RTPR-HSE-PRO-0008. This procedure does not reproduce those requirements. All personnel working at height must comply with RTPR-HSE-PRO-0008 in full.

Where a task at height requires tools that are not covered by the standard tethering arrangements defined in RTPR-HSE-PRO-0008, the work supervisor raises the matter with the HSESC team before work commences and documents an approved control in the relevant JHA. A permit is raised via the site PTW system before commencing work, refer RTPR-HSE-PRO-0021.

7  Definitions and Acronyms

The following terms are used throughout this procedure. Where a term is also used in other project HSESC procedures, the definition below applies consistently across the system.

| Term | Definition |
| --- | --- |
| HSESC | Health, Safety, Environment and Social (Community) — the integrated management framework governing occupational health, safety, environmental protection and community engagement on Rio Tinto owner-led projects. |
| HSE | Health, Safety and Environment — the discipline and organisational function responsible for managing workplace health, safety and environmental risks. |
| PRO | Procedure — a documented set of instructions that specifies the manner in which an activity or process is to be performed, forming part of the HSESC management system. |
| REG | Register — a controlled document used to record, track and manage specific items, equipment, personnel or compliance data within the management system. |
| RT | Rio Tinto — the owner organisation responsible for the project and accountable for the application of all project HSESC standards and procedures. |
| RTPR | Rio Tinto Project — the project-level designation applied to all documents, procedures and registers issued under the owner-led project management system. |
| DROPS | Dropped Objects Prevention Scheme — a structured programme and industry framework for identifying, assessing and controlling the risk of objects falling from height, including the tethering and securing of tools and equipment used at elevation. |
| PPE | Personal Protective Equipment — equipment worn or used by an individual to minimise exposure to hazards that could cause serious injury or illness, including items such as helmets, gloves, eye protection and fall-arrest devices. |
| SWL | Safe Working Load — the maximum load that a tool, lifting device or piece of equipment is designed to support or handle safely under specified operating conditions, as determined by the manufacturer or a competent engineer. |
| TAG | Tool and Equipment Tag — a physical identifier attached to a tool or piece of equipment that records inspection status, certification date, next due date and fitness-for-service condition, used to communicate serviceability at the point of use. |
| OOC | Out of Commission — a status designation applied to defective or non-compliant tools and equipment that have been withdrawn from service pending repair, recertification or disposal. |
| WAH | Working at Height — any work activity performed at an elevation where a person or object could fall a distance liable to cause personal injury or where dropped objects could cause injury to persons below. |
| ITP | Inspection and Test Plan — a document that specifies the inspection activities, hold points, witness points, acceptance criteria and responsible parties required to verify that equipment meets specified requirements before being placed into service. |
| NCR | Non-Conformance Report — a formal record raised when a tool, piece of equipment or associated process fails to meet specified requirements, triggering corrective action and defect management processes. |
| CAPA | Corrective Action and Preventive Action — a systematic process used within incident and defect management to identify root causes of non-conformances and implement measures to prevent recurrence. |
| [INSERT: add project-specific terms] | [INSERT: definition] |

8  References and Source Information

This section lists the internal and external documents this procedure relies on. This section remains in the procedure once published. Where a referenced document is updated, this procedure is reviewed to confirm the reference remains current.

| Document | Reference | Type |
| --- | --- | --- |
| Working at Height Procedure (including DROPS Programme — tool securing at height) | RTPR-HSE-PRO-0008 | Internal procedure |
| Training Procedure (equipment inspection competency requirements) | RTPR-HSE-PRO-0004 | Internal procedure |
| Incident Management Procedure (defective equipment incidents and reporting) | RTPR-HSE-PRO-0009 | Internal procedure |
| Electrical Equipment Register (quarterly inspection records) | RTPR-HSE-REG-0010 | Internal procedure |
| Rio Tinto Risk Management Standard | RT Risk Management Standard | External RT |
| Rio Tinto Risk Management Standard — Appendix C, Section C-1 (Consequence and Likelihood Criteria) | RT Risk Management Standard, Appendix C, C-1 | External RT |
| Dropped Objects Prevention Scheme — Industry Guidelines and Best Practice Framework | DROPS Guidelines (current edition) | External framework |
| AS/NZS 4600 — Cold-Formed Steel Structures (applicable to tool and equipment fabrication standards where relevant) | AS/NZS 4600 | External framework |
| AS/NZS 3000 — Wiring Rules (Electrical Installations) — applicable to electrical tool inspection and compliance | AS/NZS 3000 | External framework |
| ISO 45001 — Occupational Health and Safety Management Systems — Requirements with Guidance for Use | ISO 45001:2018 | External framework |
| [INSERT: further referenced document, where applicable] | [INSERT: reference] | [INSERT: type] |

9  Project Localisation Edits

This section captures the project-specific edits made to this document during preparation. This section stays in the document once published and is updated at each document revision. Record all additions, deletions, and wording changes from the original template.

Records of wording changes

| Description of the Change | Section |
| --- | --- |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |

Records of new content additions

| Description of the Change | Section |
| --- | --- |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |

Records of content removal

| Description of the Change | Section |
| --- | --- |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |
| [INSERT: description of change] | [INSERT: section] |

REMOVABLE SECTION — Source Requirements Traceability. Delete this entire section before the document is issued for approval. It must not appear in any approved version of this document. Table A records requirements addressed in this procedure. Table B records requirements not fully addressed, with written justification for each.

10  Source Requirements Traceability (DELETE BEFORE ISSUE)

10.1  Requirements Addressed in This Procedure

| Section | Source | Requirement | Status |
| --- | --- | --- | --- |
| [INSERT: section] | Appendix C C-2 §61.1 | Requires all lifting equipment to hold valid test certificates and all portable electrical equipment to be inspected and tagged quarterly by a certified electrician, with records maintained in a register. | Full |
| [INSERT: section] | Appendix C C-2 §61.2.1 | Prohibits specific tools and equipment on project sites under any circumstances, including improvised tools, over-centre binders, 9-inch angle grinders, sit harnesses, Saferight auto-stop rope descenders, and manual torque multipliers. | Full |
| [INSERT: section] | Appendix C C-2 §61.2.2 | Restricts certain items from use without documented project owner approval, including open-bladed knives, powered nail guns, hydro blasting equipment, personal music devices with headphones, handheld demolition saws, double-pointed podgy bars, and hardened steel sledgehammers. | Full |
| [INSERT: section] | IFC EHS General Guidelines §2.1 | Requires a systematic pre-use inspection process for all safety-critical tools and equipment and a defect management process that removes unsafe items from service immediately. | Full |

10.2  Requirements Not Fully Addressed in This Procedure

List any requirements from the external sources above that are not fully addressed in this document, with a specific written justification for each. Acceptable justifications: (a) the requirement is owned by a named standalone procedure cross-referenced here; (b) excluded by a specific business decision endorsed by project leadership, named here; or (c) addressed through an operational control outside this document, identified here.

| Requirement Not Covered | Source | Justification |
| --- | --- | --- |
| [INSERT: description of requirement not covered] | [INSERT: source and clause] | [INSERT: specific written justification] |